Adaptive Junior V2

TEC1-12706 Peltier Control — Project Walkthrough

Controller: ADJ-48-450-UR-V2  |  Peltier: TEC1-12706  |  Interface: USB-C → Linux

# 1. System Overview

The Adaptive Junior V2 is a PID-capable PWM controller that drives the TEC1-12706 Peltier module. Two K-type thermocouples measure temperature on each side of the Peltier. A Linux PC connects via USB-C and talks to the board over a plain text serial protocol at 115,200 baud.

| Component | Details |
| --- | --- |
| Controller | Adaptive Junior V2 — ADJ-48-450-UR-V2 (PID PWM, 11–48 V DC, 15 A max) |
| Peltier module | TEC1-12706 — 12 V, 6 A, 60 W, 40×40 mm |
| Temp. sensors | 2× K-type thermocouple: TC1 on hot side, TC2 on cold side |
| PC interface | USB-C (J9) → /dev/ttyACM0 or /dev/ttyUSB0, 115200 baud 8N1 |
| Control accuracy | ±0.1 °C (per user guide) |
| Temperature range | −50 °C to +250 °C setpoint |

# 2. TEC1-12706 Peltier Module

The TEC1-12706 is a standard 40×40 mm thermoelectric module. It pumps heat from one face to the other when current flows through it. Reversing current direction swaps which face is hot and which is cold — the controller handles this automatically in Bidirectional mode.

| Supply voltage | 12 V DC |
| --- | --- |
| Maximum current | 6 A |
| Maximum power | 60 W |
| Module dimensions | 40 × 40 mm |
| Max ΔT (no load) | ~68 °C |
| Polarity | Red = positive (+), Black = negative (−) |

Important: The controller output voltage closely follows the supply voltage. Use a 12 V supply to match the TEC rating. Never run the Peltier without heatsinks and thermal paste on both faces — dry running causes permanent damage within seconds.

# 3. Hardware Wiring

## 3.1 Power Input (J4)

| J4 Pin 1 | +12 V DC supply |
| --- | --- |
| J4 Pin 2 | GND (supply −) |

## 3.2 TEC Output (J5)

Wire the Peltier directly to J5. Keep wires under 50 cm to minimise resistance loss.

| J5 Pin 1 (TE OUT+) | TEC1-12706 red wire (+) |
| --- | --- |
| J5 Pin 2 (TE OUT−) | TEC1-12706 black wire (−) |

## 3.3 Thermocouples (J2)

J2 holds both K-type thermocouple inputs. Standard K-type colour coding: yellow = positive (+), red = negative (−). Polarity matters — reversed wires give wrong readings.

| J2 Pin | Signal | Sensor ID | Connect to |
| --- | --- | --- | --- |
| 4 | TC1+ | Sensor E | TC1 hot side — yellow wire |
| 3 | TC1− | Sensor E | TC1 hot side — red wire |
| 2 | TC2+ | Sensor F | TC2 cold side — yellow wire |
| 1 | TC2− | Sensor F | TC2 cold side — red wire |

TC1 maps to register 107 (Sensor E) and TC2 maps to register 108 (Sensor F) in the serial protocol.

## 3.4 Fan(s) (J3)

Connect a heatsink cooling fan to J3 Fan 1. A second fan for the cold side is optional. To power fans directly from the board supply, fit jumper P12 (Fan 1) or P13 (Fan 2).

| J3 Pin 1 | Fan 1 GND (−) |
| --- | --- |
| J3 Pin 2 | Fan 1 PWR (+) |
| J3 Pin 3 | Fan 1 TACH (3/4-wire fans only) |
| J3 Pin 4 | Fan 1 PWM signal (4-wire fans only) |

## 3.5 USB Connection (J9)

Connect a USB-C cable from J9 to your Linux PC. Power the board on before opening the serial port.

Find the port on Linux:

ls /dev/tty{USB,ACM}*

dmesg | tail -20

If you get a permission error, add yourself to the dialout group:

sudo usermod -aG dialout $USER

Then log out and back in.

# 4. First-Time Setup

Follow this sequence on every new board or after a factory reset.

- Wire power, TEC, thermocouples, and fan. Do not power on yet.

- Connect USB-C to the Linux PC.

- Power on the board — the green LED should illuminate.

- Enable both thermocouples: write register 106 = 48 (binary 110000, bits 4 and 5).

- Set TC1 as the control sensor: write register 109 = 4.

- Set output mode to Bidirectional: write register 3 = 2.  (Control mode must be OFF first.)

- Configure Fan 1: register 39 = 1 (2-wire), register 40 = 1 (PWM), register 41 = 80 (80% demand).

- Set temperature alarms: hot side high limit 65 °C, cold side low limit −10 °C.

# 5. Control Modes

## 5.1 OFF (mode 0)

Output drive completely disabled. Use this mode to change output direction or clear a shutdown alarm.

## 5.2 Manual (mode 1)

Open-loop control. The setpoint is entered as 0–100% of full output, with no sensor feedback. Useful for testing polarity and verifying TEC direction before enabling closed-loop control.

| Setpoint 0% | Zero output |
| --- | --- |
| Setpoint 50% | Zero output (bidirectional null point) |
| Setpoint < 50% | Heating (negative drive) |
| Setpoint > 50% | Cooling (positive drive) |
| Setpoint 100% | Full cooling |

## 5.3 PID (mode 3) — Recommended

Closed-loop temperature control. The controller continuously adjusts PWM output based on feedback from the selected control sensor to hold the setpoint.

| Register | Parameter | Typical start | Notes |
| --- | --- | --- | --- |
| 4 | Setpoint (°C) | 20.0 | Target temperature |
| 5 | P gain | Use auto-tune | Proportional term |
| 6 | I gain | Use auto-tune | Integral term |
| 7 | D gain | Use auto-tune | Differential term |
| 109 | Control sensor | 4 = TC1 hot side | 4=TC1, 5=TC2 |

## 5.4 Auto-Tune (mode 4)

The board drives the TEC through full positive and negative step cycles, measures the thermal response, and automatically calculates P, I, and D values. Takes 5–10 minutes. The system must be fully assembled and able to handle maximum drive power safely during this process. Supervise until complete.

# 6. Serial Protocol Reference

Connection settings: 115,200 baud, 8 data bits, 1 stop bit, no parity.  Commands start with $ and end with CR+LF.

## 6.1 Commands

| Command | Example | Description |
| --- | --- | --- |
| $ID | → ID=Junior V2... | Get device identity string |
| $REG n | $REG 107 → TC1 temp | Read register n |
| $REG n=x | $REG 4=25.0 | Write value x to register n |
| $RUN | — | Start output drive unconditionally |
| $STOP | — | Stop output drive (shutdown) |

## 6.2 Key Registers — Live Readings (read-only)

| Register | Read/Write | Unit | Description |
| --- | --- | --- | --- |
| 107 | Read | °C | TC1 temperature — hot side (Sensor E) |
| 108 | Read | °C | TC2 temperature — cold side (Sensor F) |
| 78 | Read | V | TEC bridge voltage |
| 80 | Read | A | TEC bridge current |
| 82 | Read | % | PWM output value |
| 83 | Read | V | Board supply voltage |
| 1 | Read | — | Status register (bitmask — see user guide Table 9) |

## 6.3 Key Registers — Control (read/write)

| Register | Read/Write | Unit | Description |
| --- | --- | --- | --- |
| 2 | R/W | — | Control mode: 0=OFF 1=Manual 2=Thermostat 3=PID 4=AutoTune |
| 3 | R/W | — | Output drive: 0=+ve only 1=−ve only 2=Bidirectional 3=TRIAC |
| 4 | R/W | °C | Setpoint (−50 to +250) |
| 5 | R/W | — | P (proportional) gain |
| 6 | R/W | — | I (integral) gain |
| 7 | R/W | — | D (differential) gain |
| 106 | R/W | — | Sensor enable bitmask (48 = TC1 + TC2 both enabled) |
| 109 | R/W | — | Control sensor: 4=TC1 (Sensor E) 5=TC2 (Sensor F) |

# 7. Linux Control Software

## 7.1 Installation

Install the required Python libraries:

pip install pyserial rich --break-system-packages

## 7.2 Running the UI

python3 peltier_control.py

python3 peltier_control.py --port /dev/ttyACM0

The UI auto-detects the first available /dev/ttyACM* or /dev/ttyUSB* port. It polls the board every second and displays a live three-panel readout:

- Temperatures panel — TC1 (hot side), TC2 (cold side), and ΔT between them

- Control panel — current mode, setpoint, P/I/D gains, control sensor

- Bridge/TEC panel — PWM output %, TEC voltage, TEC current, supply voltage, status flags

## 7.3 Interactive Commands

| m | Set control mode (OFF / Manual / PID / Auto-Tune) |
| --- | --- |
| s | Set setpoint in °C |
| p / i / d | Set P, I, D gain values individually |
| o | Set output drive mode (must be in OFF mode first) |
| c | Set control sensor (TC1 hot side or TC2 cold side) |
| e | Enable TC1 + TC2 sensors (register 106 = 48) |
| r | RUN — start output drive |
| x | STOP — shutdown output |
| q | Quit the application |

# 8. Typical Workflow

- Assembly: Mount TEC between heatsinks with thermal paste on both faces. Attach thermocouples to hot and cold heatsinks, wire fans.

- Power up: Connect USB-C to Linux PC, then apply 12 V supply. Green LED confirms normal operation.

- Verify sensors: Run the control UI. Press 'e' to enable TC1+TC2. Confirm both temperatures read plausible values near room temperature.

- Test direction: Set mode to Manual, setpoint 60%. TC1 should warm; setpoint 40% should cool. This confirms TEC polarity is correct.

- Auto-tune: Set mode to OFF, then Auto-Tune. Wait 5–10 minutes while the board characterises the thermal system and sets P/I/D automatically.

- PID control: Set mode to PID, enter your target setpoint. Monitor both TC temperatures in the live display.

- Fine-tune: Adjust P/I/D interactively if overshoot or oscillation is observed. Increase D to dampen oscillation; reduce P if overshooting.

# 9. Safety Checklist

Complete before first power-on and after any hardware change:

- Supply voltage confirmed at 12 V DC (matches TEC1-12706 maximum)

- Output wires (J5 to TEC) shorter than 50 cm

- Heatsink and fan fitted on hot side before running

- Thermal paste applied on both TEC faces

- Hot-side high-temperature alarm enabled (limit ≤ 65 °C)

- Cold-side low-temperature alarm enabled (limit ≥ −10 °C)

- Over-current alarm enabled on bridge output

- Both thermocouple readings verified before switching to PID mode